GroLab Pro Open Grow Shop Link: https://opengrow.pt/shop/grolab-kits/27-grolab-pro

|  | REF: COMBO003 GroLab™ Pro Kit is the most complete automation kit you'll ever get. It allows to fully monitor and automate the climate, lighting, irrigation, pH correction, nutrients dosing and all the tank management tasks, while constantly checking your soil moisture and feeding your plants only when it's necessary. This kit also provides the necessary tools to create security systems to react and/or notify if any issues arises. Remote control, real time notifications, IP cameras integration... Are just some of the perks you get with GroLab™. Includes: 1 - Starter Kit (1 GroNode and 1 PowerBot) 1 - TankBot Kit 1 - SoilBot Kit 1 - GroLab™ software, manuals and video tutorials. *This kit contains one GroNode, allowing you to further expand your control just by adding extra modules. GroNode can control a max of 4 PowerBots, 4 TankBots and 4 SoilBots. |
| --- | --- |

# What does GroLab™ Pro Kit allow me to control?

Example Configuration

Get rid of all the timers, triplets and messy cables hanging in your grow room, get control of your solution nutrients and pH, and even save water and fertilizers.

With this configuration you get precise control of your lamps, configuring the growing days from 18h/6h to 12h/12h in one click, allowing even to make an 8h/8h day if you wish so.

The temperature and humidity probe enable the control of the ventilation, lowering the consumption for this device while optimizing the climate in your grow. Connecting two water pumps to PowerBot allows the control of irrigation in two distinct grows, whether hydroponics or soil.

In soil, thanks to Soilbot you can create alarms for minimum moisture levels that trigger a timed drip irrigation, and in hydroponics you can use them to warn you if the bed is getting too dry, meaning a pump might be broken. Not only it can warn you, it can also take action and stop the pump. The same happens with the detection of a flood or a leak. Notifications, selective shutdown, are just some of the procedures you can configure.

TankBot will ensure the prefect health of the nutrients solution, automatically adjusting your pH, while dispensing the correct amount of nutrients every time a refill is made. It can even handle a full tank refill on its own. Due to the ability to control solenoids, you can use TankBot as an irrigation path creator, with one single water pump feed different grows at different times.

TankBot is also a powerful security tool, since you can sample any switch sensor, like smoke or motion detectors, allowing you for example to shut down all your room, or even the whole system and send you an e-mail when, someone gets to near, or smoke is detected.

This configuration still provides an extra output in TankBot, that you may use them for whatever device you may find suitable, from a solenoid to a peristaltic, or a power relay that handles lights, ventilators, etc.

GroLab™ Pro Kit stores every sensor data and output state, providing a track of your grows from seed to fruit, enabling you to maximize your growth and efficiency!

This is a scheme designed to show some of the GroLab™ Pro Kit possibilities, nevertheless, you can rearrange the devices, and even replace them, with any other device you may find useful, lights, fog makers, fans, dehumidifiers etc. If you need more sensors or outputs, or just to control another growing area, you just need to add extra modules to this kit (GroNode supports up to 4 modules of each type) and expand your control with new features.

| BALASTS & LAMPS | EXTRACTORS | WATER PUMPS | VAPORIZERS | CO2 DISPENSERS | ... |
| --- | --- | --- | --- | --- | --- |

| PERISTALTIC PUMPS | ANY KIND OF SOLENOID | DC WATER PUMPS | POWER RELAY | ... |
| --- | --- | --- | --- | --- |

| PH PROBE | EC PROBE |
| --- | --- |

| LEVEL SENSORS | SMOKE DETECTORS | MOTION DETECTORS | BUTTONS |
| --- | --- | --- | --- |

| AIR TEMP/HUMI PROBE | SOIL/AIR/WATER TEMPERATURE PROBE | MOISTURE PROBE | FLOOD DETECTOR |
| --- | --- | --- | --- |

But, what do I get with a GroLab™ Pro Kit?

This is the exact package that will arrive at your door step. It comes with GroNode, a PowerBot, a TankBot and a SoilBot, containing all the necessary cables, sensors and connectors. It also comes with a USB Pen with the GroLab™ software, instruction manuals and video tutorials.

It can be installed in no time and will fully automate your grow in a matter of minutes.

|  | ### PERIODICAL UPDATES GroLab™ Software is always under development. If a newer version is available, the system will automatically update. The same happens for any of the included modules, this means that your GroLab™ Pro Kit will have access to any improvement our development team implements in the near future. |
| --- | --- |
|  | ### MODULAR Remember, one GroNode supports up to 4 PowerBots, 4 SoilBots and 4 TankBots, and you can spread them between 4 different areas with two different grows each. Whenever you need a new vegetative, cloning or flowering room, you can just extend your GroLab™ system by adding as many extra GroLab™ modules GroNode supports. |
|  | ### SUPPORT Your plants are our concern! GroLab™ team is here to help you with any doubt, configuration or troubleshoot you may need. We’re committed to your complete satisfaction, and we won't let a kit or peripheral left of use. By phone, e-mail, support ticket, Skype, just contact us and in less than 24 hours someone will come to your aid. |

GroNode

|  | ### THE HEART & BRAIN OF THE OPERATION A state of the art growing unit that interacts with other GroLab™ modules simulating the operation of a brain. It is capable of executing hundreds of instructions and to store a large amount of data allowing a detailed analysis about the life cycle of the user’s plants. GroNode will do all the hard work for you and will notify you if a risk situation arises. The only limit to GroNode is your imagination. |
| --- | --- |

|  | ### Autonomous GroNode is a powerful computer that can do hundreds of tasks by itself. You don't need any PC or other third-party device connected to GroNode for it to continue working. |
| --- | --- |
|  | ### Monitor & Analysis Using GroLab™ Software the user can monitor and analyze the climate variables and the devices current state in real time. Charts, grow(s) overview and manual control are just some perks you can use. |
|  | ### Notifications When you provide Internet connection to GroNode it can send real time alerts and updates to your e-mail, keeping you updated about the state of your grow(s) anywhere, anytime. |
|  | ### Remote Control When connecting the GroNode to a router with an Internet connection, it allows you to activate the GroLab™ system's remote control. This feature grants user access from anywhere at any time through an easy-to-use software. Allowing to fully control all the modules. |
|  | ### Programmable Procedures There are two main types of programmable procedures, alarms and schedules. GroNode has the ability to store and execute up to 100 of each type allowing a precise and extensive automation of any grow(s). |
|  | ### Modular GroNode is the core of the GroLab™ modular architecture, it is capable of controlling up to 4 modules of each type (PowerBot, TankBot and SoilBot). This makes it easily adaptable to any grow regardless of its size, type, growing medium or growing system. |
|  | ### Secure All GroLab™ communications are protected by several security layers, securing your valuable data. By using Open Grow™ private server, your E-mail notifications are also secured, not letting anyone pinpoint your grow(s). |
|  | ### Data-Logging GroNode has its own memory that is capable of storing hundreds of thousands of data. With data-logging and data-visualization features, it is possible to do a detailed analysis about the life cycle of the user’s plants. |
|  | ### Cloud Upload Connecting GroNode to the Internet allows the user to configure a cloud server to periodically upload the data-log from all sensors/devices. |

| GroNode Specifications |  |
| --- | --- |
| Dimensions | 91mm x 91mm x 28.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset, NET Reset, FW Update |
| Power Supply | USB - 5VDC 1000mA |
| Connections | USB 2.1 type B Ethernet LAN RJ45 RP-SMA female |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| USB Power Adapter Included | Yes (Type A 230AC-5VDC) |
| Ethernet LAN Cable Included | Yes |
| Ethernet LAN Cable Length | 1.52 meters |
| Inter-Module Communication | Radio Frequency - 2.4GHz |
| Battery | CR2032 Lithium 3V 250mAh |
| Storage Memory | 2MB |
| Audiovisual Indicators | Buzzer single tone LED RGB |
| Warranty | 2-years limited hardware warranty |

GRONODE INSTRUCTION MANUAL

PowerBot

|  | ### THE COMPLETE ALL IN ONE POWER MODULE PowerBot allows you to connect and control the electrical peripherals of your growing environment. This powerful unit monitors and automates all the basic elements of your grow like temperature, humidity, air flow, lighting and irrigation. Create safety protocols & procedures to avoid overheating and minimize the damage in case of fire. With PowerBot every variable is under your direct control, enabling you to maximize your growth and efficiency! |
| --- | --- |

|  | ### Power Supplier PowerBot provides power to all peripherals of your growing environment. It has 4 universal outlets that supports up to 2300W (per outlet and in total) or unlimited power when using an external electrical contactor. |
| --- | --- |
|  | ### Lighting Automate any lighting system connected to PowerBot. With our advanced lightning system, you are not bound to the 24 hours combined day, your grow(s) can now have 20 hours of combined day/night, or even less if you wish so. |
|  | ### Climate The included temperature & humidity sensor and the capacity to independently control the electrical devices, allows you to perform a precise climate automation in your grow(s). |
|  | ### Irrigation From drip systems to hydroponics, GroLab™ schedule capabilities allow you to preset the irrigation for any kind of system. |
|  | ### Manual Control This module comes with 4 external switches that give the user the ability to manually control all connected peripherals at any time on the spot. |
|  | ### LED Indicator Its design features 4 LEDs that indicates which elements are ON. Useful to visually understand what is happening on your grow(s). |
|  | ### Safety Protocols & Procedures PowerBot allows you to create safety protocols & procedures to avoid risk situations or even react to them in order to minimize the damage. The cool-down feature prevents damage to devices that need some time to cool down before turning them on (HPS Lamps). |
|  | ### Wireless Communication between Modules PowerBot communicates with GroNode through radio frequency with a range of 25 meters indoors (depending on obstacles) and 100 meters in open spaces. This makes it easy to install the PowerBot on your growing area or even on the outside. |

| PowerBot Specifications |  |
| --- | --- |
| Dimensions | 236.4mm x 91.4mm x 46.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset, NET Reset, FW Update |
| Power Supply | EU - 230v AC US - 120v AC |
| Fuse Included | Yes (250V 16A) |
| Cord Included | Yes (Universal) |
| Cord Length | 2 meters |
| Connections | USB 2.1 type B RP-SMA female Power Entry IEC 320-C14 2.5mm 4-Pin male |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| Outlets | 4 x Universal |
| Outlet Maximum Power | EU - 2300W(10A) US - 1200W(10A) |
| Total Maximum Power | EU - 2300W(10A) US - 1200W(10A) |
| Temperature & Humidity Sensor Included | Yes |
| Warranty | 2-years limited hardware warranty |

POWERBOT INSTRUCTION MANUAL

Temperature & Humidity Sensor Specifications

| Dimensions | 27mm x 58.75mm x 13.30mm |
| --- | --- |
| Exterior | Casing: Plastic Colors: White |
| Cable Length | 2 meters |
| Temperature Range | -40ºC ~ 80ºC |
| Temperature Precision | ±0.5ºC |
| Humidity Range | 0% ~ 99.9% |
| Humidity Precision | ±2% |
| Compatibility | GroLab™ PowerBot GroLab™ UserBot |
| Warranty | 1-year limited hardware warranty |

TankBot

|  | ### THE POWERFUL TANK MANAGER TankBot is the most versatile and precise module in the GroLab™ family. It could be described as a complete controller that is able to cover all the aspects of tank management. However, this description is not enough because TankBot is much more than that. PH & water temperature regulation, nutrients dosing, water level management and tank refill/drain are just some examples of the TankBot capabilities. |
| --- | --- |

|  | ### Power Supplier TankBot can provide power to all peripherals of your growing environment. It has 4 connectors that support 12VDC~24VDC actuators allowing a wide variety of devices from small water pumps, peristaltic pumps, relays, power contactors, etc. |
| --- | --- |
|  | ### PH Monitor & Adjustment This module gives you the possibility to deep analysis, monitor and control the pH. The GroLab™ system programmable procedures offer several options to regulate pH based on the user needs. |
|  | ### EC Monitor & Analysis TankBot also supports EC probe extending the monitor and control of your water tank to another level. GroLab™ system can notify you and even react based on electrical conductivity of your water. |
|  | ### Water Temperature TankBot provides all you need to continuously regulate the water temperature ensuring the optimum conditions for your plants to grow. |
|  | ### Nutrients Dosing Connecting peristaltic pumps on TankBot opens the door to the nutrients dosing domain. GroLab™ system provides all the necessary tools to calibrate the peristaltic pumps and to create procedures to pump the exact amount of milliliters required. |
|  | ### Speed Customization TankBot provides the capability to customize device speed. This feature can improve the precision even with low cost devices. |
|  | ### Advance Irrigation Thanks to the capability of handling actuators of 12v, it is possible to create impressive irrigation systems that can independently feed multiple grows. |
|  | ### Tank Level Management This module supports 4 extra switch sensors of your choice like water level sensors, allowing to continuously monitor the water level of your tanks. With the right peripherals connected to the TankBot, like solenoid valves or water pumps, user can create procedures to automatically drain and refill, allowing a complete tank maintenance. |
|  | ### Safety Protocols & Procedures TankBot allows you to create safety protocols & procedures to avoid risk situations or even react to them in order to minimize the damage. Fires and intrusions are some examples of risk situations this module can handle. The cool-down feature prevents damage to devices that need some time to cool down before turning them on. |
|  | ### LED Indicator ### Its design features a LED that indicates if the module is currently powered on and the connection state with GroNode (the system's core module). |
|  | ### Wireless Communication between Modules TankBot communicates with GroNode (the system's core module) through radio frequency with a range of 25 meters indoors (depending on obstacles) and 100 meters in open spaces. This makes it easy to install the TankBot close to the water tank. |

| TankBot Specifications |  |
| --- | --- |
| Dimensions | 91mm x 108.9mm x 28.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset |
| Power Supply | 12V/2A |
| Power Supply Adapter Included | Yes |
| Power Supply Adapter Cable Length | 1 meter |
| Connections | USB 2.1 type B RP-SMA female Power Entry 12V/1A 24V/2A 4 x 12v Actuators (2.5mm 4-Pin male) 4 x Switch Sensor (2.5mm 4-Pin male) Temperature Sensor (2.5mm 4-Pin male) PH Connector (BNC) EC Connector (BNC 50Ω) |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| USB Power Adapter Included | Yes (Type A 230AC-5VDC/2A) |
| Sensors Included | PH Probe |
| Connector Plug Included | Yes (4 x 2.5mm 4-Pin female) |
| Inter-Module Communication | Radio Frequency - 2.4GHz |
| Visual Indicators | LED |
| Warranty | 2-years limited hardware warranty |

TANKBOT INSTRUCTION MANUAL

| PH Probe Specifications |  |
| --- | --- |
| Dimensions | 140mm (length) x 12mm (diameter) |
| Exterior | Casing: Plastic Colors: Green |
| Cable Length | 3 meters |
| Measure Range | 0PH ~ 14PH |
| Measuring Temperature | 0ºC ~ 80ºC |
| Resolution | 0.01PH |
| Accuracy | ±0.01PH |
| Zero | 7±0.25PH |
| Alkali Error | 15mV |
| PTS | >98.5 |
| Response Time | <1min |
| Internal Resistance | <250MΩ |
| Repeatability | <0.017 |
| Noise | <0.017 |
| Warranty | 1-year limited hardware warranty |

| Temperature Sensor Specifications |  |
| --- | --- |
| Dimensions | 50mm (length) x 10mm (diameter) |
| Exterior | Casing: Stainless Steel Colors: Silver |
| Cable Length | 2 meters |
| Waterproof | Yes |
| Temperature Range | -40ºC ~ 110ºC |
| Temperature Precision | ±0.5ºC |
| Warranty | 1-year limited hardware warranty |

SoilBot

|  | ### THE VERSATILE SUBSTRATE ANALYZER SoilBot is a rigorous and versatile substrate analyzer, it is another module that makes part of the GroLab™ family. It is capable to completely monitor the substrate of your plants regardless of whether it is soil, rock wool, coconut or a recurrent dipping of roots in water. Supports up to 4 moisture sensors giving you the possibility to monitor 4 different groups of plants at same time. When combined with PowerBot or TankBot allows you to automate their irrigation processes, keeping the perfect moisture for your plants avoiding water wasting and floods. SoilBot has also support for 2 temperature sensors allowing you a complete substrate analysis and 2 flood detectors that makes the system able to react/notify in case of flood detection. Take the control of your plants substrate with this versatile module! |
| --- | --- |

|  | ### Moisture SoilBot has the ability to monitor any substrate's moisture regardless of whether it is soil, rock wool, coconut or a recurrent dipping of roots in water. That's right SoilBot can also be used in a hydroponic set up! It supports up to 4 moisture sensors giving you the possibility to monitor 4 different groups of plants at same time. When combined with PowerBot or TankBot allows you to automate their irrigation processes, keeping the perfect moisture for your plants avoiding water wasting and floods. |
| --- | --- |
|  | ### Temperature With this versatile module even the substrate temperature can be fully monitored. SoilBot supports 2 temperature sensors that greatly extends its substrate analysis capabilities. By combining SoilBot with PowerBot or Tankbot, it is possible to create procedures to keep substrate temperature at the perfect value for your plants to continue to grow healthier. |
|  | ### Flood Prevention & Detection Equipped with flood detection system, it will quickly detect water leaks, excess water and automatically activate the security features that the user has programmed to minimize damages and even notify you of any anomaly. |
|  | ### LED Indicator Its design features a LED that indicates if the module is currently powered on and the connection state with GroNode (the system's core module). |
|  | ### Wireless Communication between Modules SoilBot communicates with GroNode (the system's core module) through radio frequency with a range of 25 meters indoors (depending on obstacles) and 100 meters in open spaces. This makes it easy to install the SoilBot near the plants. |

| SoilBot Specifications |  |
| --- | --- |
| Dimensions | 91mm x 91mm x 28.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset |
| Power Supply | USB - 5VDC 1000mA |
| Connections | USB 2.1 type B RP-SMA female Power Entry 12V/1A 24V/2A 4 x Moisture Sensor (2.5mm 4-Pin male) 2 x Temperature Sensor (2.5mm 4-Pin male) 2 x Flood Detector (2.5mm 4-Pin male) |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| USB Power Supply Adapter Included | Yes |
| Sensors Included | 2 x Moisture Sensor Temperature Sensor Flood Detector |
| Inter-Module Communication | Radio Frequency - 2.4GHz |
| Visual Indicators | LED |
| Warranty | 2-years limited hardware warranty |

SOILBOT INSTRUCTION MANUAL

| Moisture Sensor Specifications |  |
| --- | --- |
| Dimensions | 13mm x 99.89mm x 2.5mm |
| Exterior | Material: FR-4 Colors: Pigment Green |
| Cable Length | 2 meters |
| Moisture Range | 5% ~ 100% 2 x |
| Moisture Precision | Variable (calibration dependent) |
| Compatibility | GroLab SoilBot |
| Warranty | 1-year limited hardware warranty |

| Temperature Sensor Specifications |  |
| --- | --- |
| Dimensions | 50mm (length) x 10mm (diameter) |
| Exterior | Casing: Stainless Steel Colors: Silver |
| Cable Length | 2 meters |
| Waterproof | Yes |
| Temperature Range | -40ºC ~ 110ºC |
| Temperature Precision | ±0.5ºC |
| Warranty | 1-year limited hardware warranty |

Peripherals/Sensors Compatibility

# What devices and sensors can I connect to PowerBot?

In its core, PowerBot can sample Air Temperature and Air Humidity, and comes with a probe for this purpose. It also has an internal temperature sensor for security purposes.

AIR TEMP/HUMI PROBE

Besides that, PowerBot comes with 4 controlled outputs that can turn on or off most of your grow devices.

### Outputs

Each PowerBot output can turn on/off any device that operates at 230VAC/110VAC and does not consume more than 10A. Here are some examples of devices you can connect to your PowerBot.

| BALLASTS & LAMPS | EXTRACTORS | WATER PUMPS | VAPORIZERS | CO2 DISPENSERS | ... |
| --- | --- | --- | --- | --- | --- |

# What devices and sensors can I connect to TankBot?

In its core, TankBot can sample EC/pH and Water Temperature, these are static inputs available in any TankBot, this kit comes by default with a pH Probe and Water Temperature Probe.

| WATER TEMPERATURE PROBE | PH PROBE | EC PROBE |
| --- | --- | --- |

Besides that, TankBot comes with 4 controlled outputs and 4 switched inputs. All of them fully configurable via the GroLab™ Software.

Outputs

Each TankBot output can turn on/off any device that works at 12VDC that does not consume more than 6W. Here are some examples of devices you can connect to your TankBot.

| PERISTALTIC PUMPS | ANY KIND OF SOLENOID | DC WATER PUMP | POWER RELAY | ... |
| --- | --- | --- | --- | --- |

INPUTS

Each TankBot can sense up to 4 switched inputs, what to connect to these inputs is up to you, from water level detectors, smoke detectors or even simple buttons, GroLab™ allows you to sample these sensors and act accordingly. Below you can see some of the switch sensors you can use.

| LEVEL SENSORS | SMOKE DETECTORS | MOTION DETECTORS | BUTTONS |
| --- | --- | --- | --- |

# What sensors can I connect to SoilBot?

SoilBot is a sample only purpose module, in its core, it can sample 2 Soil/Air/Water Temperature, up to 4 Moisture sensors and also 2 Flood detectors.

The SoilBot Kit comes by default with 2 Moisture Sensors, 1 Temperature Sensor and 1 Flood Detector.

| SOIL/AIR/WATER TEMPERATURE PROBE | MOISTURE PROBE | FLOOD DETECTOR |
| --- | --- | --- |

All these sensors are supplied with a 4 pin quick connector that connects directly to SoilBot, make it possible to assemble the SoilBot Kit in a matter of minutes.

Software GroLab™

GroLab™ Software is the control booth of the system. The main purpose of this software is to allow you to fully configure the whole system, providing all the tools to customize modules and devices settings matching the functionalities of any agricultural growing system. Track your grow(s) progress through IP cameras and a variant of data inputs, featuring graphs, historical data, and trends. All sensors/actuators data can easily be exported to a friendly file for an external deep analysis. Using a third-party device capable of cloud data logging, GroLab™ system can periodically upload data to that device in the JSON format. Thanks to the remote control feature, this easy-to-use software allows you to keep controlling and monitoring your grow(s) like if you were there.

SOFTWARE DOWNLOAD

| GroLab™ Software System Requirements |  |
| --- | --- |
| CPU | 1.6GHz |
| Memory | 1GB |
| Disk Space | 500MB |
| Operating System | Windows 7, 8, 8.1, 10 |
| Screen Resolution | 1366 x 768 |