GroLab Doser Kit Open Grow Shop Link: https://opengrow.pt/shop/grolab-kits/43-grolab-doser

|  | REF: COMBO004 GroLab™ Doser Kit is a powerful grow controller that allows you to fully monitor and automate the feeding solution of your plants, nutrients control, pH correction, water level warnings, auto tank refill, are some functions of this kit. This kit also provides the necessary tools to create security systems to react and/or notify if any issues arises. Remote control, real time notifications, IP cameras integration... are just some of the perks you get with GroLab™. Includes: 1 - GroNode 1 - TankBot Kit 3 - Peristaltic Pumps 2 - Horizontal Level Sensors 1 - GroLab™ software, manuals and video tutorials. *This kit contains one GroNode, allowing you to further expand your control just by adding extra modules. GroNode can control a max of 4 PowerBots, 4 TankBots and 4 SoilBots. |
| --- | --- |

# What does GroLab™ Doser Kit allow me to control?

This configuration allows you to automatically dispense the precise amounts of fertilizer A & B to your solution whenever the tank is refilled or whenever you schedule it.

PH is automatically set to the value you define by automatically compensating raise or falls in your solution. And the water levels are constantly analyzed through the level sensors available in this kit.

TankBot is also a powerful security tool, since you can sample any switch sensor, like smoke or motion detectors, allowing you for example to shut down all your room, or even the whole system and send you an e-mail when someone gets too near, or smoke is detected. Notifications, selective shutdown, are just some of the procedures you can configure.

You still get a free output and 2 inputs to use the way you find suitable.

GroLab™ Doser Kit stores every sensor data and output state, providing a track of your grows from seed to fruit, enabling you to maximize your growth and efficiency!

This is a scheme designed to show some of the GroLab™ Doser Kit possibilities, nevertheless, you can rearrange the devices, and even replace them, with any other device you may find useful, peristaltics, solenoids, power relays, etc. If you need more sensors or outputs, or just to control another growing area, you just need to add extra modules to this kit (GroNode supports up to 4 modules of each type) and expand your control with new features.

| PERISTALTIC PUMPS | ANY KIND OF SOLENOID | DC WATER PUMPS | POWER RELAY | ... |
| --- | --- | --- | --- | --- |

| SOIL/AIR/WATER TEMPERATURE PROBE | PH PROBE | EC PROBE |
| --- | --- | --- |

| LEVEL SENSORS | SMOKE DETECTORS | MOTION DETECTORS | BUTTONS |
| --- | --- | --- | --- |

But, what do I get with a GroLab™ Doser Kit?

This is the exact package that will arrive at your door step. It comes with our GroNode, a TankBot and all the necessary cables, sensors and connectors, and additional you get 3 peristaltic pumps and 2 level sensors, allowing you to control the nutrients and pH in your water tank. It also comes with a USB Pen with the GroLab™ Software, instruction manuals and video tutorials.

TankBot is capable of handling much more than what this kit brings, you can still add another peristaltic pumps, solenoid, or smoke detectors, buttons or water level sensors, just go to our Peripherals & Extras section and improve your GroLab™ Doser Kit.

It can be installed in no time and will fully automate your grow in a matter of minutes.

|  | ### PERIODICAL UPDATES GroLab™ Software is always under development. If a newer version is available, the system will automatically update. The same happens for any of the included modules, this means that your GroLab™ Doser Kit will have access to any improvement our development team implements in the near future. |
| --- | --- |
|  | ### MODULAR Remember, one GroNode supports up to 4 PowerBots, 4 SoilBots and 4 TankBots, and you can spread them between 4 different areas with two different grows each. Whenever you need a new vegetative, cloning or flowering room, you can just extend your GroLab™ system by adding as many extra GroLab™ modules GroNode supports. |
|  | ### SUPPORT Your plants are our concern! GroLab™ team is here to help you with any doubt, configuration or troubleshoot you may need. We’re committed to your complete satisfaction, and we won't let a kit or peripheral left of use. By phone, e-mail, support ticket, Skype, just contact us and in less than 24 hours someone will come to your aid. |

GroNode

|  | ### THE HEART & BRAIN OF THE OPERATION A state of the art growing unit that interacts with other GroLab™ modules simulating the operation of a brain. It is capable of executing hundreds of instructions and to store a large amount of data allowing a detailed analysis about the life cycle of the user’s plants. GroNode will do all the hard work for you and will notify you if a risk situation arises. The only limit to GroNode is your imagination. |
| --- | --- |

|  | ### Autonomous GroNode is a powerful computer that can do hundreds of tasks by itself. You don't need any PC or other third-party device connected to GroNode for it to continue working. |
| --- | --- |
|  | ### Monitor & Analysis Using GroLab™ Software the user can monitor and analyze the climate variables and the devices current state in real time. Charts, grow(s) overview and manual control are just some perks you can use. |
|  | ### Notifications When you provide Internet connection to GroNode it can send real time alerts and updates to your e-mail, keeping you updated about the state of your grow(s) anywhere, anytime. |
|  | ### Remote Control When connecting the GroNode to a router with an Internet connection, it allows you to activate the GroLab™ system's remote control. This feature grants user access from anywhere at any time through an easy-to-use software. Allowing to fully control all the modules. |
|  | ### Programmable Procedures There are two main types of programmable procedures, alarms and schedules. GroNode has the ability to store and execute up to 100 of each type allowing a precise and extensive automation of any grow(s). |
|  | ### Modular GroNode is the core of the GroLab™ modular architecture, it is capable of controlling up to 4 modules of each type (PowerBot, TankBot and SoilBot). This makes it easily adaptable to any grow regardless of its size, type, growing medium or growing system. |
|  | ### Secure All GroLab™ communications are protected by several security layers, securing your valuable data. By using Open Grow™ private server, your E-mail notifications are also secured, not letting anyone pinpoint your grow(s). |
|  | ### Data-Logging GroNode has its own memory that is capable of storing hundreds of thousands of data. With data-logging and data-visualization features, it is possible to do a detailed analysis about the life cycle of the user’s plants. |
|  | ### Cloud Upload Connecting GroNode to the Internet allows the user to configure a cloud server to periodically upload the data-log from all sensors/devices. |

| GroNode Specifications |  |
| --- | --- |
| Dimensions | 91mm x 91mm x 28.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset, NET Reset, FW Update |
| Power Supply | USB - 5VDC 1000mA |
| Connections | USB 2.1 type B Ethernet LAN RJ45 RP-SMA female |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| USB Power Adapter Included | Yes (Type A 230AC-5VDC) |
| Ethernet LAN Cable Included | Yes |
| Ethernet LAN Cable Length | 1.52 meters |
| Inter-Module Communication | Radio Frequency - 2.4GHz |
| Battery | CR2032 Lithium 3V 250mAh |
| Storage Memory | 2MB |
| Audiovisual Indicators | Buzzer single tone LED RGB |
| Warranty | 2-years limited hardware warranty |

GRONODE INSTRUCTION MANUAL

TankBot

|  | ### THE POWERFUL TANK MANAGER TankBot is the most versatile and precise module in the GroLab™ family. It could be described as a complete controller that is able to cover all the aspects of tank management. However, this description is not enough because TankBot is much more than that. PH & water temperature regulation, nutrients dosing, water level management and tank refill/drain are just some examples of the TankBot capabilities. |
| --- | --- |

|  | ### Power Supplier TankBot can provide power to all peripherals of your growing environment. It has 4 connectors that support 12VDC~24VDC actuators allowing a wide variety of devices from small water pumps, peristaltic pumps, relays, power contactors, etc. |
| --- | --- |
|  | ### PH Monitor & Adjustment This module gives you the possibility to deep analysis, monitor and control the pH. The GroLab™ system programmable procedures offer several options to regulate pH based on the user needs. |
|  | ### EC Monitor & Analysis TankBot also supports EC probe extending the monitor and control of your water tank to another level. GroLab™ system can notify you and even react based on electrical conductivity of your water. |
|  | ### Water Temperature TankBot provides all you need to continuously regulate the water temperature ensuring the optimum conditions for your plants to grow. |
|  | ### Nutrients Dosing Connecting peristaltic pumps on TankBot opens the door to the nutrients dosing domain. GroLab™ system provides all the necessary tools to calibrate the peristaltic pumps and to create procedures to pump the exact amount of milliliters required. |
|  | ### Speed Customization TankBot provides the capability to customize device speed. This feature can improve the precision even with low cost devices. |
|  | ### Advance Irrigation Thanks to the capability of handling actuators of 12v, it is possible to create impressive irrigation systems that can independently feed multiple grows. |
|  | ### Tank Level Management This module supports 4 extra switch sensors of your choice like water level sensors, allowing to continuously monitor the water level of your tanks. With the right peripherals connected to the TankBot, like solenoid valves or water pumps, user can create procedures to automatically drain and refill, allowing a complete tank maintenance. |
|  | ### Safety Protocols & Procedures TankBot allows you to create safety protocols & procedures to avoid risk situations or even react to them in order to minimize the damage. Fires and intrusions are some examples of risk situations this module can handle. The cool-down feature prevents damage to devices that need some time to cool down before turning them on. |
|  | ### LED Indicator ### Its design features a LED that indicates if the module is currently powered on and the connection state with GroNode (the system's core module). |
|  | ### Wireless Communication between Modules TankBot communicates with GroNode (the system's core module) through radio frequency with a range of 25 meters indoors (depending on obstacles) and 100 meters in open spaces. This makes it easy to install the TankBot close to the water tank. |

| TankBot Specifications |  |
| --- | --- |
| Dimensions | 91mm x 108.9mm x 28.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset |
| Power Supply | 12V/2A |
| Power Supply Adapter Included | Yes |
| Power Supply Adapter Cable Length | 1 meter |
| Connections | USB 2.1 type B RP-SMA female Power Entry 12V/1A 24V/2A 4 x 12v Actuators (2.5mm 4-Pin male) 4 x Switch Sensor (2.5mm 4-Pin male) Temperature Sensor (2.5mm 4-Pin male) PH Connector (BNC) EC Connector (BNC 50Ω) |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| USB Power Adapter Included | Yes (Type A 230AC-5VDC/2A) |
| Sensors Included | PH Probe |
| Connector Plug Included | Yes (4 x 2.5mm 4-Pin female) |
| Inter-Module Communication | Radio Frequency - 2.4GHz |
| Visual Indicators | LED |
| Warranty | 2-years limited hardware warranty |

TANKBOT INSTRUCTION MANUAL

| PH Probe Specifications |  |
| --- | --- |
| Dimensions | 140mm (length) x 12mm (diameter) |
| Exterior | Casing: Plastic Colors: Green |
| Cable Length | 3 meters |
| Measure Range | 0PH ~ 14PH |
| Measuring Temperature | 0ºC ~ 80ºC |
| Resolution | 0.01PH |
| Accuracy | ±0.01PH |
| Zero | 7±0.25PH |
| Alkali Error | 15mV |
| PTS | >98.5 |
| Response Time | <1min |
| Internal Resistance | <250MΩ |
| Repeatability | <0.017 |
| Noise | <0.017 |
| Warranty | 1-year limited hardware warranty |

| Temperature Sensor Specifications |  |
| --- | --- |
| Dimensions | 50mm (length) x 10mm (diameter) |
| Exterior | Casing: Stainless Steel Colors: Silver |
| Cable Length | 2 meters |
| Waterproof | Yes |
| Temperature Range | -40ºC ~ 110ºC |
| Temperature Precision | ±0.5ºC |
| Warranty | 1-year limited hardware warranty |

Peripherals/Sensors Compatibility

# What devices and sensors can I connect to TankBot?

In its core, TankBot can sample EC/pH and Water Temperature, these are static inputs available in any TankBot, this kit comes by default with a pH Probe and Water Temperature Probe.

| WATER TEMPERATURE PROBE | PH PROBE | EC PROBE |
| --- | --- | --- |

Besides that, TankBot comes with 4 controlled outputs and 4 switched inputs. All of them fully configurable via the GroLab™ Software.

Outputs

Each TankBot output can turn on/off any device that works at 12VDC that does not consume more then 6W. Here are some examples of devices you can connect to your TankBot.

| PERISTALTIC PUMPS | ANY KIND OF SOLENOID | DC WATER PUMP | POWER RELAY | ... |
| --- | --- | --- | --- | --- |

INPUTS

Each TankBot can sense up to 4 switched inputs, what to connect to these inputs is up to you, from water level detectors, smoke detectors or even simple buttons, GroLab™ allows you to sample these sensors and act accordingly. Below you can see some of the switch sensors you can use.

| LEVEL SENSORS | SMOKE DETECTORS | MOTION DETECTORS | BUTTONS |
| --- | --- | --- | --- |

Software GroLab™

GroLab™ Software is the control booth of the system. The main purpose of this software is to allow you to fully configure the whole system, providing all the tools to customize modules and devices settings matching the functionalities of any agricultural growing system. Track your grow(s) progress through IP cameras and a variant of data inputs, featuring graphs, historical data, and trends. All sensors/actuators data can easily be exported to a friendly file for an external deep analysis. Using a third-party device capable of cloud data logging, GroLab™ system can periodically upload data to that device in the JSON format. Thanks to the remote control feature, this easy-to-use software allows you to keep controlling and monitoring your grow(s) like if you were there.

SOFTWARE DOWNLOAD

| GroLab™ Software System Requirements |  |
| --- | --- |
| CPU | 1.6GHz |
| Memory | 1GB |
| Disk Space | 500MB |
| Operating System | Windows 7, 8, 8.1, 10 |
| Screen Resolution | 1366 x 768 |