GroLab PB 17kW - 24 x 600W Open Grow Shop Link: https://opengrow.pt/shop/perifericos-extras/185-grolab-pb-17kw-24-x-600w

|  | REF: COMP0027 The GroLab™ PowerBoard comes with 17kW of total Power and 6 lines of control. Supporting up to 24x600W or 12x1000W Lamps it's ideal to expand the number of lights you can handle without any hassle. Controlled with 230VAC, you can use it with GroLab™ PowerBot or any kind of commercial or industrial timer. Possible configurations: - 12x1000W: 12 lamps of 1000W, 2 lamps per line. - 24x600W: 24 lamps of 600W, 4 lamps per line. Compatibility POWERBOT. |
| --- | --- |

# How to Install GroLab™ PowerBoard?

GroLab PowerBoard comes with a 230VAC Male plug that is used to control the contactor inside the PowerBoard.

To install the PowerBoard, you need to take the cover of the board out...

In the bottom connectors present inside the PowerBoard, you need to connect your Live wires, Neutral and Earth as presented in the picture.

The maximum input power is 75A, so make sure you use the proper cables to power your GroLab PowerBoard.

The recommended cable for Live/Neutral connection must be lower or equal to AWG10.

| Specifications of GroLab™ PowerBoard 17kW |  |
| --- | --- |
| Operating Voltage | 230VAC |
| Control Voltage | 230VAC |
| Absolute Max. Power | 17kW |
| Recommended Max Power | 14.4kW |
| Differential Residual Current Protection | 300mA |
| Lines | 6 (4 tomadas por linha) |
| Max Power per line | 3680W |
| Max Power recommended per line | 2400W |
| Compatibility | GroLab PowerBot 230VAC Timer |
| Warranty | 2-year warranty limited to hardware |