Starter Kit Open Grow Shop Link: https://opengrow.pt/shop/grolab-kits/8-grolab-starter-kit

|  | REF: KIT0001 GroLab™ Starter Kit is a powerful grow controller that fully monitors and automates all the basic elements of any grow. It can handle lighting, climate and irrigation, all in the same system. Remote control, real time notifications, IP cameras integration... are just some of the perks you get with GroLab™. Includes: 1 - GroNode 1 - PowerBot 1 - GroLab™ software, manuals and video tutorials. *This kit contains one GroNode, allowing you to further expand your control just by adding extra modules. GroNode can control a max of 4 PowerBots, 4 TankBots and 4 SoilBots. |
| --- | --- |

# What does GroLab™ Starter Kit allows me to control?

Example Configuration

Get rid of all the timers, triplets and messy cables hanging in your grow room.

With this configuration you get precise control of your lamps, configuring the growing days from 18h/6h to 12h/12h in one click, allowing even to make an 8h/8h day if you wish so.

The temperature and humidity probe enables the control of the ventilation, lowering the consumption for this device while optimizing the climate in your grow.

Connecting a water pump to PowerBot allows the control of irrigation (hydroponics or soil), from the basic weekdays selection (one hour in the morning every other day), to the enhanced recurrent schedule (5m every hour). The GroLab™ software allows the creation of schedules to feed any kind of grow type with extreme precision.

This configuration still provides an extra output that you may use for whatever device you may find suitable, from a second water pump, more light power or even a humidifier.

GroLab™ Starter Kit stores every sensor data and output state, providing a track of your grows from seed to fruit, enabling you to maximize your growth and efficiency!

This is a scheme designed to show some of the GroLab™ Starter Kit possibilities, nevertheless, you can rearrange the devices, and even replace them, with any other device you may find useful, lights, fog makers, fans, dehumidifiers etc. If you need more sensors or outputs, or just to control another growing area, you just need to add extra modules to this kit (GroNode supports up to 4 modules of each type) and expand your control with new features.

| BALASTS & LAMPS | EXTRACTORS | WATER PUMPS | VAPORIZERS | CO2 DISPENSERS | ... |
| --- | --- | --- | --- | --- | --- |

AIR TEMP/HUMI PROBE

But, what do I get with a GroLab™ Starter Kit?

This is the exact package that will arrive at your door step. It comes with a GroNode, a PowerBot and all the necessary cables, sensors and connectors. It also comes with a USB Pen with the GroLab™ software, instruction manuals and video tutorials.

It can be installed in no time and will fully automate your grow in a matter of minutes.

|  | ### PERIODICAL UPDATES GroLab™ Software is always under development. If a newer version is available, the system will automatically update. The same happens for any of the included modules, this means that your GroLab™ Starter Kit will have access to any improvement our development team implements in the near future. |
| --- | --- |
|  | ### MODULAR Remember, one GroNode supports up to 4 PowerBots, 4 SoilBots and 4 TankBots, and you can spread them between 4 different areas with two different grows each. Whenever you need a new vegetative, cloning or flowering room, you can just extend your GroLab™ system by adding as many extra GroLab™ modules GroNode supports. |
|  | ### SUPPORT Your plants are our concern! GroLab™ team is here to help you with any doubt, configuration or troubleshoot you may need. We’re committed to your complete satisfaction, and we won't let a kit or peripheral left of use. By phone, e-mail, support ticket, Skype, just contact us and in less than 24 hours someone will come to your aid. |

GroNode

|  | ### THE HEART & BRAIN OF THE OPERATION A state of the art growing unit that interacts with other GroLab™ modules simulating the operation of a brain. It is capable of executing hundreds of instructions and to store a large amount of data allowing a detailed analysis about the life cycle of the user’s plants. GroNode will do all the hard work for you and will notify you if a risk situation arises. The only limit to GroNode is your imagination. |
| --- | --- |

|  | ### Autonomous GroNode is a powerful computer that can do hundreds of tasks by itself. You don't need any PC or other third-party device connected to GroNode for it to continue working. |
| --- | --- |
|  | ### Monitor & Analysis Using GroLab™ Software the user can monitor and analyse the climate variables and the devices current state in real time. Charts, grow(s) overview and manual control are just some perks you can use. |
|  | ### Notifications When you provide Internet connection to GroNode it can send real time alerts and updates to your e-mail, keeping you updated about the state of your grow(s) anywhere, anytime. |
|  | ### Remote Control When connecting the GroNode to a router with an Internet connection, it allows you to activate the GroLab™ system's remote control. This feature grants user access from anywhere at any time through an easy-to-use software. Allowing to fully control all the modules. |
|  | ### Programmable Procedures There are two main types of programmable procedures, alarms and schedules. GroNode has the ability to store and execute up to 100 of each type allowing a precise and extensive automation of any grow(s). |
|  | ### Modular GroNode is the core of the GroLab™ modular architecture, it is capable of controlling up to 4 modules of each type (PowerBot, TankBot and SoilBot). This makes it easily adaptable to any grow regardless of its size, type, growing medium or growing system. |
|  | ### Secure All GroLab™ communications are protected by several security layers, securing your valuable data. By using Open Grow™ private server, your E-mail notifications are also secured, not letting anyone pinpoint your grow(s). |
|  | ### Data-Logging GroNode has its own memory that is capable of storing hundreds of thousands of data. With data-logging and data-visualization features, it is possible to do a detailed analysis about the life cycle of the user’s plants. |
|  | ### Cloud Upload Connecting GroNode to the Internet allows the user to configure a cloud server to periodically upload the data-log from all sensors/devices. |

| GroNode Specifications |  |
| --- | --- |
| Dimensions | 91mm x 91mm x 28.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset, NET Reset, FW Update |
| Power Supply | USB - 5VDC 1000mA |
| Connections | USB 2.1 type B Ethernet LAN RJ45 RP-SMA female |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| USB Power Adapter Included | Yes (Type A 230AC-5VDC) |
| Ethernet LAN Cable Included | Yes |
| Ethernet LAN Cable Length | 1.52 meters |
| Inter-Module Communication | Radio Frequency - 2.4GHz |
| Battery | CR2032 Lithium 3V 250mAh |
| Storage Memory | 2MB |
| Audiovisual Indicators | Buzzer single tone LED RGB |
| Warranty | 2-years limited hardware warranty |

GRONODE INSTRUCTION MANUAL

PowerBot

|  | ### THE COMPLETE ALL IN ONE POWER MODULE PowerBot allows you to connect and control the electrical peripherals of your growing environment. This powerful unit monitors and automates all the basic elements of your grow like temperature, humidity, air flow, lighting and irrigation. Create safety protocols & procedures to avoid overheating and minimize the damage in case of fire. With PowerBot every variable is under your direct control, enabling you to maximize your growth and efficiency! |
| --- | --- |

|  | ### Power Supplier PowerBot provides power to all peripherals of your growing environment. It has 4 universal outlets that supports up to 2300W (per outlet and in total) or unlimited power when using an external electrical contactor. |
| --- | --- |
|  | ### Lighting Automate any lighting system connected to PowerBot. With our advanced lightning system, you are not bound to the 24 hours combined day, your grow(s) can now have 20 hours of combined day/night, or even less if you wish so. |
|  | ### Climate The included temperature & humidity sensor and the capacity to independently control the electrical devices, allows you to perform a precise climate automation in your grow(s). |
|  | ### Irrigation From drip systems to hydroponics, GroLab™ schedule capabilities allow you to preset the irrigation for any kind of system. |
|  | ### Manual Control This module comes with 4 external switches that give the user the ability to manually control all connected peripherals at any time on the spot. |
|  | ### LED Indicator Its design features 4 LEDs that indicates which elements are ON. Useful to visually understand what is happening on your grow(s). |
|  | ### Safety Protocols & Procedures PowerBot allows you to create safety protocols & procedures to avoid risk situations or even react to them in order to minimize the damage. The cool-down feature prevents damage to devices that need some time to cool down before turning them on (HPS Lamps). |
|  | ### Wireless Communication between Modules PowerBot communicates with GroNode through radio frequency with a range of 25 meters indoors (depending on obstacles) and 100 meters in open spaces. This makes it easy to install the PowerBot on your growing area or even on the outside. |

| PowerBot Specifications |  |
| --- | --- |
| Dimensions | 236.4mm x 91.4mm x 46.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset, NET Reset, FW Update |
| Power Supply | EU - 230v AC US - 120v AC |
| Fuse Included | Yes (250V 16A) |
| Cord Included | Yes (Universal) |
| Cord Length | 2 meters |
| Connections | USB 2.1 type B RP-SMA female Power Entry IEC 320-C14 2.5mm 4-Pin male |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| Outlets | 4 x Universal |
| Outlet Maximum Power | EU - 2300W(10A) US - 1200W(10A) |
| Total Maximum Power | EU - 2300W(10A) US - 1200W(10A) |
| Temperature & Humidity Sensor Included | Yes |
| Warranty | 2-years limited hardware warranty |

POWERBOT INSTRUCTION MANUAL

Temperature & Humidity Sensor Specifications

| Dimensions | 27mm x 58.75mm x 13.30mm |
| --- | --- |
| Exterior | Casing: Plastic Colors: White |
| Cable Length | 2 meters |
| Temperature Range | -40ºC ~ 80ºC |
| Temperature Precision | ±0.5ºC |
| Humidity Range | 0% ~ 99.9% |
| Humidity Precision | ±2% |
| Compatibility | GroLab™ PowerBot GroLab™ UserBot |
| Warranty | 1-year limited hardware warranty |

Peripherals/Sensors Compatibility

# What devices and sensors can I connect to PowerBot?

In its core, PowerBot can sample Air Temperature and Air Humidity, and comes with a probe for this purpose. It also has an internal temperature sensor for security purposes.

AIR TEMP/HUMI PROBE

Besides that, PowerBot comes with 4 controlled outputs that can turn on or off most of your grow devices.

### Outputs

Each PowerBot output can turn on/off any device that operates at 230VAC/110VAC and does not consume more than 10A. Here are some examples of devices you can connect to your PowerBot.

| BALLASTS & LAMPS | EXTRACTORS | WATER PUMPS | VAPORIZERS | CO2 DISPENSERS | ... |
| --- | --- | --- | --- | --- | --- |

Software GroLab™

GroLab™ Software is the control booth of the system. The main purpose of this software is to allow you to fully configure the whole system, providing all the tools to customize modules and devices settings matching the functionalities of any agricultural growing system. Track your grow(s) progress through IP cameras and a variant of data inputs, featuring graphs, historical data, and trends. All sensors/actuators data can easily be exported to a friendly file for an external deep analysis. Using a third-party device capable of cloud data logging, GroLab™ system can periodically upload data to that device in the JSON format. Thanks to the remote control feature, this easy-to-use software allows you to keep controlling and monitoring your grow(s) like if you were there.

SOFTWARE DOWNLOAD

| GroLab™ Software System Requirements |  |
| --- | --- |
| CPU | 1.6GHz |
| Memory | 1GB |
| Disk Space | 500MB |
| Operating System | Windows 7, 8, 8.1, 10 |
| Screen Resolution | 1366 x 768 |