GroLab TankBot Open Grow Shop Link: https://opengrow.pt/shop/grolab-modules/12-grolab-tankbot

|  | REF: KIT0003 Indispensable as a nutrients and pH controller, water tank level manager or even a security system. It can control any 12v actuators like water valves or peristaltic pumps, while also analyzing the water solution in your tank thanks to the pH, EC, Temperature and water level sensors. Refill the tank, recycle the water and even automatically control the nutrients and pH. Includes: 1 - TankBot 1 - pH Probe 1 - Water/Soil/Air temperature sensor 1 - 4-pin connectors for sensors and actuators (Pack of 4) 1 - 12V Transformer 1 - USB cable 1 - Antenna *Additional GroLab™ module, one GroNode is necessary to experience the full functionalities of TankBot. *Each GroNode can control a max of 4 TankBot's. |
| --- | --- |

TankBot

|  | ### THE POWERFUL TANK MANAGER TankBot is the most versatile and precise module in the GroLab™ family. It could be described as a complete controller that is able to cover all the aspects of tank management. However, this description is not enough because TankBot is much more than that. PH & water temperature regulation, nutrients dosing, water level management and tank refill/drain are just some examples of the TankBot capabilities. |
| --- | --- |

|  | ### Power Supplier TankBot can provide power to all peripherals of your growing environment. It has 4 connectors that support 12VDC~24VDC actuators allowing a wide variety of devices from small water pumps, peristaltic pumps, relays, power contactors, etc. |
| --- | --- |
|  | ### PH Monitor & Adjustment This module gives you the possibility to deep analysis, monitor and control the pH. The GroLab™ system programmable procedures offer several options to regulate pH based on the user needs. |
|  | ### EC Monitor & Analysis TankBot also supports EC probe extending the monitor and control of your water tank to another level. GroLab™ system can notify you and even react based on electrical conductivity of your water. |
|  | ### Water Temperature TankBot provides all you need to continuously regulate the water temperature ensuring the optimum conditions for your plants to grow. |
|  | ### Nutrients Dosing Connecting peristaltic pumps on TankBot opens the door to the nutrients dosing domain. GroLab™ system provides all the necessary tools to calibrate the peristaltic pumps and to create procedures to pump the exact amount of milliliters required. |
|  | ### Speed Customization TankBot provides the capability to customize device speed. This feature can improve the precision even with low cost devices. |
|  | ### Advance Irrigation Thanks to the capability of handling actuators of 12v, it is possible to create impressive irrigation systems that can independently feed multiple grows. |
|  | ### Tank Level Management This module supports 4 extra switch sensors of your choice like water level sensors, allowing to continuously monitor the water level of your tanks. With the right peripherals connected to the TankBot, like solenoid valves or water pumps, user can create procedures to automatically drain and refill, allowing a complete tank maintenance. |
|  | ### Safety Protocols & Procedures TankBot allows you to create safety protocols & procedures to avoid risk situations or even react to them in order to minimize the damage. Fires and intrusions are some examples of risk situations this module can handle. The cool-down feature prevents damage to devices that need some time to cool down before turning them on. |
|  | ### LED Indicator ### Its design features a LED that indicates if the module is currently powered on and the connection state with GroNode (the system's core module). |
|  | ### Wireless Communication between Modules TankBot communicates with GroNode (the system's core module) through radio frequency with a range of 25 meters indoors (depending on obstacles) and 100 meters in open spaces. This makes it easy to install the TankBot close to the water tank. |

| TankBot Specifications |  |
| --- | --- |
| Dimensions | 91mm x 108.9mm x 28.7mm |
| Exterior | Casing: Stainless Steel Colors: Silver Physical buttons: Reset |
| Power Supply | 12V/2A |
| Power Supply Adapter Included | Yes |
| Power Supply Adapter Cable Length | 1 meter |
| Connections | USB 2.1 type B RP-SMA female Power Entry 12V/1A 24V/2A 4 x 12v Actuators (2.5mm 4-Pin male) 4 x Switch Sensor (2.5mm 4-Pin male) Temperature Sensor (2.5mm 4-Pin male) PH Connector (BNC) EC Connector (BNC 50Ω) |
| Antenna Included | Yes |
| USB Cable Included | Yes (Type B-A) |
| USB Cable Length | 2 meters |
| USB Power Adapter Included | Yes (Type A 230AC-5VDC/2A) |
| Sensors Included | PH Probe |
| Connector Plug Included | Yes (4 x 2.5mm 4-Pin female) |
| Inter-Module Communication | Radio Frequency - 2.4GHz |
| Visual Indicators | LED |
| Warranty | 2-years limited hardware warranty |

TANKBOT INSTRUCTION MANUAL

| PH Probe Specifications |  |
| --- | --- |
| Dimensions | 140mm (length) x 12mm (diameter) |
| Exterior | Casing: Plastic Colors: Green |
| Cable Length | 3 meters |
| Measure Range | 0PH ~ 14PH |
| Measuring Temperature | 0ºC ~ 80ºC |
| Resolution | 0.01PH |
| Accuracy | ±0.01PH |
| Zero | 7±0.25PH |
| Alkali Error | 15mV |
| PTS | >98.5 |
| Response Time | <1min |
| Internal Resistance | <250MΩ |
| Repeatability | <0.017 |
| Noise | <0.017 |
| Warranty | 1-year limited hardware warranty |

| Temperature Sensor Specifications |  |
| --- | --- |
| Dimensions | 50mm (length) x 10mm (diameter) |
| Exterior | Casing: Stainless Steel Colors: Silver |
| Cable Length | 2 meters |
| Waterproof | Yes |
| Temperature Range | -40ºC ~ 110ºC |
| Temperature Precision | ±0.5ºC |
| Warranty | 1-year limited hardware warranty |

Peripherals/Sensors Compatibility

# What devices and sensors can I connect to TankBot?

In its core, TankBot can sample EC/pH and Water Temperature, these are static inputs available in any TankBot, this kit comes by default with a pH Probe and Water Temperature Probe.

| WATER TEMPERATURE PROBE | PH PROBE | EC PROBE |
| --- | --- | --- |

Besides that, TankBot comes with 4 controlled outputs and 4 switched inputs. All of them fully configurable via the GroLab™ Software.

Outputs

Each TankBot output can turn on/off any device that works at 12VDC that does not consume more then 6W. Here are some examples of devices you can connect to your TankBot.

| PERISTALTIC PUMPS | ANY KIND OF SOLENOID | DC WATER PUMP | POWER RELAY | ... |
| --- | --- | --- | --- | --- |

INPUTS

Each TankBot can sense up to 4 switched inputs, what to connect to these inputs is up to you, from water level detectors, smoke detectors or even simple buttons, GroLab™ allows you to sample these sensors and act accordingly. Below you can see some of the switch sensors you can use.

| LEVEL SENSORS | SMOKE DETECTORS | MOTION DETECTORS | BUTTONS |
| --- | --- | --- | --- |